IPSEC SITE TO SITE VPN

CLASS TASK - 1

TRANSPORT MODE CONFIGURATION

R1

crypto isakmp policy 10

encryption 3des

hash md5

authentication pre-share

group 5

lifetime 50000

crypto isakmp key cisco address 2.2.2.1

crypto ipsec transform-set TSET esp-des esp-sha-hmac

mode transport

crypto map CMAP 10 ipsec-isakmp

set peer 2.2.2.1

set transform-set TSET

match address 101

interface Loopback0

ip address 10.1.1.1 255.255.255.255

interface FastEthernet0/0

ip address 1.1.1.1 255.255.255.0

crypto map CMAP

ip route 0.0.0.0 0.0.0.0 1.1.1.10

access-list 101 permit ip host 1.1.1.1 host 2.2.2.1

INTERNET

interface FastEthernet0/0
ip address 1.1.1.10 255.255.255.0

interface FastEthernet0/1

ip address 2.2.2.10 255.255.255.0

R2

crypto isakmp policy 10

encryption 3des

hash md5

authentication pre-share

group 5

lifetime 25000

crypto isakmp key cisco address 1.1.1.1

crypto ipsec transform-set TSET esp-des esp-sha-hmac

mode transport

crypto map CMAP 10 ipsec-isakmp

set peer 1.1.1.1

set transform-set TSET

match address 101

interface Loopback0

ip address 10.1.2.1 255.255.255.255

interface FastEthernet0/0

ip address 2.2.2.1 255.255.255.0

crypto map CMAP

ip route 0.0.0.0 0.0.0.0 2.2.2.10

access-list 101 permit ip host 2.2.2.1 host 1.1.1.1

Note:

- Even though we can manually configure the mode of the tunnel, the router assigns the mode based on the interesting traffic it matched with the crypto ACL.

When the interesting traffic is between the loopbacks – Tunnel Mode (Protects user generated traffic)

When the interesting traffic is between the tunnel peer’s – Transport Mode (Protects router generated traffic)

- For the Phase 1 tunnel to come up all the parameters of the ISAKMP policy should match with the peer except for the lifetime of the Phase 1 tunnel.

TUNNEL MODE CONFIGURATION

R1

crypto isakmp policy 10

encryption 3des

hash md5

authentication pre-share

group 5

lifetime 50000

crypto isakmp key cisco address 2.2.2.1

crypto ipsec transform-set TSET esp-des esp-sha-hmac

mode tunnel

crypto map CMAP 10 ipsec-isakmp

set peer 2.2.2.1

set transform-set TSET

match address 101

interface Loopback0

ip address 10.1.1.1 255.255.255.255

interface FastEthernet0/0

ip address 1.1.1.1 255.255.255.0

crypto map CMAP

ip route 0.0.0.0 0.0.0.0 1.1.1.10

access-list 101 permit ip host 10.1.1.1 host 10.1.2.1

INTERNET

interface FastEthernet0/0
ip address 1.1.1.10 255.255.255.0

interface FastEthernet0/1

ip address 2.2.2.10 255.255.255.0

R2

crypto isakmp policy 10

encryption 3des

hash md5

authentication pre-share

group 5

lifetime 25000

crypto isakmp key cisco address 1.1.1.1

crypto ipsec transform-set TSET esp-des esp-sha-hmac

mode tunnel

crypto map CMAP 10 ipsec-isakmp

set peer 1.1.1.1

set transform-set TSET

match address 101

interface Loopback0

ip address 10.1.2.1 255.255.255.255

interface FastEthernet0/0

ip address 2.2.2.1 255.255.255.0

crypto map CMAP

ip route 0.0.0.0 0.0.0.0 2.2.2.10

access-list 101 permit ip host 10.1.2.1 host 10.1.1.1