IPSEC SITE TO SITE VPN

ASSIGNMENT - 1

TASK:

- Configure an IPSEC Site to Site (Lan to Lan) tunnel between R1 & R2 using the following parameters

Phase 1                                    Phase 2                             Mode

PSK – cisco123                       ESP-3DES                        Transport

AES – 256                               MD5

SHA

DH2

- Configure an IPSEC Site to Site (Lan to Lan) tunnel between R1 & R3 using the following parameters

Phase 1                                   Phase 2                             Mode

PSK – ccie123                        AH                                    Tunnel

AES – 256                              MD5

SHA

DH2

SOLUTION:

TASK 1

R1

int lo0

ip add 10.1.1.1 255.255.255.255

int fa0/0

ip add 1.1.1.1 255.255.255.0

no shut

ip route 0.0.0.0 0.0.0.0 1.1.1.10

access-list 101 permit ip host 1.1.1.1 host 2.2.2.1

crypto isakmp policy 10

encr aes 256

hash sha

authentication pre-share

group 2

lifetime 86400

crypto isakmp key cisco123 address 2.2.2.1

crypto ipsec transform-set R1R2-TSET esp-3des esp-md5-hmac

mode transport

crypto map R1R2-CMAP 10 ipsec-isakmp

set peer 2.2.2.1

set transform-set R1R2-TSET

match address 101

int fa0/0

crypto map R1R2-CMAP

TASK 2

access-list 101 permit ip host 10.1.1.1 host 10.1.3.1

crypto isakmp key ccie123 address 3.3.3.1

crypto ipsec transform-set R1R3-TSET ah-md5-hmac

mode tunnel

crypto map R1R2-CMAP 20 ipsec-isakmp

set peer 3.3.3.1

set transform-set R1R3-TSET

match address 101

Note : Same policy, Different Transform Set, Same Crypto map but with different

Sequence number (as we can apply only one crypto map per interface)

TASK 1

R2

int lo0

ip add 10.1.2.1 255.255.255.255

int fa0/0

ip add 2.2.2.1 255.255.255.0

no shut

ip route 0.0.0.0 0.0.0.0 2.2.2.10

access-list 101 permit ip host 10.1.3.1 host 10.1.1.1

crypto isakmp policy 10

encr aes 256

hash sha

authentication pre-share

group 2

lifetime 86400

crypto isakmp key cisco123 address 1.1.1.1

crypto ipsec transform-set R1R2-TSET esp-3des esp-md5-hmac

mode transport

crypto map R1R2-CMAP 10 ipsec-isakmp

set peer 1.1.1.1

set transform-set R1R2-TSET

match address 101

int fa0/0

crypto map R1R2-CMAP

INTERNET

int fa0/0

ip add 1.1.1.10 255.255.255.0

no shut

int fa0/1

ip add 2.2.2.10 255.255.255.0

no shut

int fa1/0

ip add 3.3.3.10 255.255.255.0

no shut

TASK 2

R3

int lo0

ip add 10.1.3.1 255.255.255.255

int fa0/0

ip add 3.3.3.1 255.255.255.0

no shut

ip route 0.0.0.0 0.0.0.0 3.3.3.10

access-list 101 permit ip host 3.3.3.1 host 1.1.1.1

crypto isakmp policy 10

encr aes 256

hash sha

authentication pre-share

group 2

lifetime 86400

crypto isakmp key ccie123 address 1.1.1.1

crypto ipsec transform-set R1R3-TSET ah-md5-hmac

mode tunnel

crypto map R1R2-CMAP 20 ipsec-isakmp

set peer 1.1.1.1

set transform-set R1R3-TSET

match address 102

int fa0/0

crypto map R1R2-CMAP