GRE OVER IPSEC USING AH RUNNING EIGRP

A COMPARISON BETWEEN TUNNEL & TRANSPORT MODE HEADERS

ASSIGNMENT - 10

Note: We use AH to do the comparison between the tunnel and transport mode headers as AH does not encrypt the packet and the header information is clearly visible in wire-shark. But if we would have used ESP then none of the headers could be seen as they will be encrypted.

AH USING TRANSPORT MODE

R1

crypto isakmp policy 10

authentication pre-share

crypto isakmp key cisco address 20.1.1.1

crypto ipsec transform-set TSET ah-sha-hmac

mode transport

crypto ipsec profile GRE-IPSEC

set transform-set TSET

interface Loopback0

ip address 1.1.1.1 255.255.255.255

interface Tunnel0

ip address 172.16.1.1 255.255.255.0

tunnel source FastEthernet0/0

tunnel destination 20.1.1.1

tunnel protection ipsec profile GRE-IPSEC

interface FastEthernet0/0

ip address 10.1.1.1 255.255.255.0

router eigrp 100

network 1.1.1.0 0.0.0.255

network 172.16.1.0 0.0.0.255

ip route 20.1.1.0 255.255.255.0 10.1.1.10

R2

crypto isakmp policy 10

authentication pre-share

crypto isakmp key cisco address 10.1.1.1

crypto ipsec transform-set TSET ah-sha-hmac

mode transport

crypto ipsec profile GRE-IPSEC

set transform-set TSET

interface Loopback0

ip address 2.2.2.2 255.255.255.255

interface Tunnel0

ip address 172.16.1.2 255.255.255.0

tunnel source FastEthernet0/0

tunnel destination 10.1.1.1

tunnel protection ipsec profile GRE-IPSEC

interface FastEthernet0/0

ip address 20.1.1.1 255.255.255.0

router eigrp 100

network 2.2.2.0 0.0.0.255

network 172.16.0.0

network 172.16.1.0 0.0.0.255

ip route 10.1.1.0 255.255.255.0 20.1.1.10

INTERNET

interface FastEthernet0/0

ip address 10.1.1.10 255.255.255.0

interface FastEthernet0/1

ip address 20.1.1.10 255.255.255.0

AH USING TUNNEL MODE

R1

crypto ipsec transform-set TSET ah-sha-hmac

mode tunnel – even if we don’t give this command the default will be tunnel mode only

R2

crypto ipsec transform-set TSET ah-sha-hmac

mode tunnel

Note: We cannot advertise the tunnel network and the physical network using the same protocol. So we advertise the physical network using static routes and the tunnel networks using EIGRP

GRE over IPSEC can operate in both tunnel mode and transport mode but transport mode is most preferred solution as it eliminates the 20 bytes additional header produced in tunnel mode.

AH USING TRANSPORT MODE PACKET

AH USING TUNNEL MODE PACKET