MGRE USING EIGRP

ASSIGNMENT - 11

R1

interface Loopback0

ip address 1.1.1.1 255.255.255.255

interface Tunnel0

ip address 172.16.1.1 255.255.255.0

no ip redirects

no ip next-hop-self eigrp 100

ip nhrp authentication cisco

ip nhrp map multicast dynamic

ip nhrp network-id 99

no ip split-horizon eigrp 100

tunnel source FastEthernet0/0

tunnel mode gre multipoint

tunnel key 123

interface FastEthernet0/0

ip address 10.1.1.1 255.255.255.0

router eigrp 100

network 1.1.1.1 0.0.0.0

network 172.16.1.0 0.0.0.255

no auto-summary

ip route 20.1.1.0 255.255.255.0 10.1.1.10

ip route 30.1.1.0 255.255.255.0 10.1.1.10

R3

interface Loopback0

ip address 2.2.2.2 255.255.255.255

interface Tunnel0

ip address 172.16.1.2 255.255.255.0

ip nhrp authentication cisco

ip nhrp map 172.16.1.1 10.1.1.1

ip nhrp map multicast 10.1.1.1

ip nhrp network-id 99

ip nhrp nhs 172.16.1.1

tunnel source FastEthernet0/0

tunnel destination 10.1.1.1

tunnel key 123

no ip split-horizon eigrp 100

interface FastEthernet0/0

ip address 20.1.1.1 255.255.255.0

router eigrp 100

network 2.2.2.2 0.0.0.0

network 172.16.1.0 0.0.0.255

no auto-summary

ip forward-protocol nd

ip route 10.1.1.0 255.255.255.0 20.1.1.10

ip route 30.1.1.0 255.255.255.0 20.1.1.10

R4

interface Loopback0

ip address 3.3.3.3 255.255.255.255

interface Tunnel0

ip address 172.16.1.3 255.255.255.0

ip nhrp authentication cisco

ip nhrp map 172.16.1.1 10.1.1.1     - this is to register with the HUB

ip nhrp map multicast 10.1.1.1      - this is to support multicast

ip nhrp network-id 99

ip nhrp nhs 172.16.1.1

tunnel source FastEthernet0/0

tunnel destination 10.1.1.1

tunnel key 123

no ip split-horizon eigrp 100       - this advertises the spoke routes

interface FastEthernet0/0

ip address 30.1.1.1 255.255.255.0

router eigrp 100

network 3.3.3.3 0.0.0.0

network 172.16.1.0 0.0.0.255

no auto-summary

ip route 10.1.1.0 255.255.255.0 30.1.1.10

ip route 20.1.1.0 255.255.255.0 30.1.1.10

INTERNET

interface FastEthernet1/0

ip address 10.1.1.10 255.255.255.0

interface FastEthernet1/1

ip address 20.1.1.10 255.255.255.0

interface FastEthernet2/0

ip address 30.1.1.10 255.255.255.0