DMVPN USING EIGRP & OSPF

ASSIGNMENT - 12

USING EIGRP

PHASE – 1 (All traffic must flow via HUB)

R1

crypto isakmp policy 10

authentication pre-share

crypto isakmp key cisco address 0.0.0.0 0.0.0.0

crypto ipsec transform-set TSET esp-des esp-sha-hmac

crypto ipsec profile DMVPN

set transform-set TSET

interface Loopback0

ip address 1.1.1.1 255.255.255.255

interface Tunnel0

ip address 172.16.1.1 255.255.255.0

ip nhrp authentication cisco

ip nhrp map multicast dynamic

ip nhrp network-id 100

no ip split-horizon eigrp 100

tunnel source FastEthernet0/0

tunnel mode gre multipoint

tunnel key 123

tunnel protection ipsec profile DMVPN

interface FastEthernet0/0

ip address 10.1.1.1 255.255.255.0

router eigrp 100

network 1.1.1.1 0.0.0.0

network 172.16.1.0 0.0.0.255

no auto-summary

ip route 20.1.1.0 255.255.255.0 10.1.1.10

ip route 30.1.1.0 255.255.255.0 10.1.1.10

R2

crypto isakmp policy 10

authentication pre-share

crypto isakmp key cisco address 10.1.1.1

crypto ipsec transform-set TSET esp-des esp-sha-hmac

crypto ipsec profile DMVPN

set transform-set TSET

interface Loopback0

ip address 2.2.2.2 255.255.255.255

interface Tunnel0

ip address 172.16.1.2 255.255.255.0

ip nhrp authentication cisco

ip nhrp map 172.16.1.1 10.1.1.1

ip nhrp network-id 100

ip nhrp nhs 172.16.1.1

tunnel source FastEthernet0/0

tunnel destination 10.1.1.1

tunnel key 123

tunnel protection ipsec profile DMVPN

interface FastEthernet0/0

ip address 20.1.1.1 255.255.255.0

router eigrp 100

network 2.2.2.2 0.0.0.0

network 172.16.1.0 0.0.0.255

no auto-summary

ip route 10.1.1.0 255.255.255.0 20.1.1.10

R3

crypto isakmp policy 10

authentication pre-share

crypto isakmp key cisco address 10.1.1.1

crypto ipsec transform-set TSET esp-des esp-sha-hmac

crypto ipsec profile DMVPN

set transform-set TSET

interface Loopback0

ip address 3.3.3.3 255.255.255.255

interface Tunnel0

ip address 172.16.1.3 255.255.255.0

ip nhrp authentication cisco

ip nhrp map 172.16.1.1 10.1.1.1

ip nhrp network-id 100

ip nhrp nhs 172.16.1.1

tunnel source FastEthernet0/0

tunnel destination 10.1.1.1

tunnel key 123

tunnel protection ipsec profile DMVPN

interface FastEthernet0/0

ip address 30.1.1.1 255.255.255.0

router eigrp 100

network 3.3.3.3 0.0.0.0

network 172.16.1.0 0.0.0.255

no auto-summary

ip route 10.1.1.0 255.255.255.0 30.1.1.10

INTERNET

interface FastEthernet1/0

ip address 10.1.1.10 255.255.255.0

interface FastEthernet1/1

ip address 20.1.1.10 255.255.255.0

interface FastEthernet2/0

ip address 30.1.1.10 255.255.255.0

PHASE – 2 (All traffic must flow DIRECTLY)

R1

int tunnel 0

no ip eigrp next-hop-self eigrp 100

R4

ip route 30.1.1.0 255.255.255.0 20.1.1.10

interface tunnel 0

no tunnel mode gre ip

tunnel mode gre multipoint

no tunnel destination 10.1.1.1

tunnel mode gre multipoint

ip nhrp map multicast 10.1.1.1

exit

no crypto isakmp key 0 cisco address 10.1.1.1

no crypto isakmp key 0 cisco address 0.0.0.0 0.0.0.0

R3

ip route 20.1.1.0 255.255.255.0 30.1.1.10

interface tunnel 0

no tunnel mode gre ip

tunnel mode gre multipoint

no tunnel destination 10.1.1.1

tunnel mode gre multipoint

ip nhrp map multicast 10.1.1.1

exit

no crypto isakmp key 0 cisco address 10.1.1.1

no crypto isakmp key 0 cisco address 0.0.0.0 0.0.0.0

PHASE – 3 (All traffic must flow DIRECTLY but with better scalability and valid CEF adjacency)

R1

interface tunnel 0

ip next-hop-self eigrp 100

ip nhrp redirect

R4

interface tunnel 0

ip nhrp shortcut

R3

interface tunnel 0

ip nhrp shortcut

USING OSPF

PHASE 1 (All traffic must flow through HUB)

R1

no router eigrp 100

router ospf 100

network 172.16.1.0 0.0.0.255 area 0

network 1.1.1.1 0.0.0.0 area 0

router id 1.1.1.1

interface tunnel 0

ip ospf network point-to-multipoint

ip ospf hello-interval 10

R4

no router eigrp 100

router ospf 100

network 172.16.1.0 0.0.0.255 area 0

network 2.2.2.2 0.0.0.0 area 0

router id 2.2.2.2

interface tunnel 0

ip ospf network point-to-point

ip ospf hello-interval 10

R3

no router eigrp 100

router ospf 100

network 172.16.1.0 0.0.0.255 area 0

network 3.3.3.3 0.0.0.0 area 0

router id 3.3.3.3

interface tunnel 0

ip ospf network point-to-point

ip ospf hello-interval 10

PHASE 2 (All traffic must flow directly)

R1

interface tunnel 0

ip ospf network broadcast

no ip ospf hello-interval 10

ip ospf priority 255

R4

interface tunnel 0

ip ospf network broadcast

no ip ospf hello-interval 10

ip ospf priority 0

R3

interface tunnel 0

ip ospf network broadcast

no ip ospf hello-interval 10

ip ospf priority 0

PHASE 3 (All traffic must flow directly with valid CEF entry and scalability)

R1

interface tunnel 0

ip ospf network point-to-multipoint

ip nhrp redirect

R4

interface tunnel 0

ip ospf network point-to-multipoint

ip nhrp shortcut

R3

interface tunnel 0

ip ospf network point-to-multipoint

ip nhrp shortcut