GET VPN USING ASA

ASSIGNMENT - 13

ASA

interface GigabitEthernet0

nameif OUTSIDE

security-level 0

ip address 10.1.1.10 255.255.255.0

no shut

interface GigabitEthernet1

nameif INSIDE

security-level 100

ip address 20.1.1.10 255.255.255.0

no shut

route INSIDE 1.1.1.1 255.255.255.255 20.1.1.1 1

route INSIDE 3.3.3.3 255.255.255.255 20.1.1.1 1

route INSIDE 30.1.1.0 255.255.255.0 20.1.1.1 1

access-list OUT-IN-REKEY permit udp host 10.1.1.1 any

access-group OUT-IN-REKEY in interface OUTSIDE

KEYSERVER

interface FastEthernet1/0

ip address 10.1.1.1 255.255.255.0

no shut

crypto isakmp policy 10

authentication pre-share

crypto isakmp key cisco address 0.0.0.0 0.0.0.0

crypto ipsec transform-set GSET esp-des esp-sha-hmac

crypto ipsec profile GET

set transform-set GSET

crypto gdoi group GDOI-GET

identity number 123

server local

rekey retransmit 10 number 2

rekey authentication mypubkey rsa KEK

rekey transport unicast

sa ipsec 1

profile GET

match address ipv4 101

replay counter window-size 1024

address ipv4 10.1.1.1

access-list 101 permit ip host 1.1.1.1 host 3.3.3.3

access-list 101 permit ip host 3.3.3.3 host 1.1.1.1

ip route 20.1.1.0 255.255.255.0 10.1.1.10

ip route 30.1.1.0 255.255.255.0 10.1.1.10

GM-1

interface FastEthernet1/0

ip address 20.1.1.1 255.255.255.0

no shut

interface FastEthernet1/1

ip address 30.1.1.1 255.255.255.0

no shut

interface Loopback0

ip address 1.1.1.1 255.255.255.255

crypto isakmp policy 10

authentication pre-share

crypto isakmp key cisco address 10.1.1.1

crypto gdoi group GDOI-GET

identity number 123

server address ipv4 10.1.1.1

crypto map CMAP 10 gdoi

set group GDOI-GET

interface FastEthernet1/1

crypto map CMAP

ip route 3.3.3.3 255.255.255.255 30.1.1.2

ip route 10.1.1.0 255.255.255.0 20.1.1.10

GM-2

interface FastEthernet1/0

ip address 30.1.1.2 255.255.255.0

no shut

interface Loopback0

ip address 3.3.3.3 255.255.255.255

crypto isakmp policy 10

authentication pre-share

crypto isakmp key cisco address 10.1.1.1

crypto gdoi group GDOI-GET

identity number 123

server address ipv4 10.1.1.1

crypto map CMAP 10 gdoi

set group GDOI-GET

interface FastEthernet1/0

crypto map CMAP

ip route 1.1.1.1 255.255.255.255 30.1.1.1

ip route 10.1.1.0 255.255.255.0 30.1.1.1

ip route 20.1.1.0 255.255.255.0 30.1.1.1