GET VPN COOP KS USING ASA

ASSIGNMENT - 14

ASA

interface GigabitEthernet0

nameif INSIDE1

security-level 100

ip address 10.1.1.10 255.255.255.0

interface GigabitEthernet1

nameif INSIDE2

security-level 99

ip address 30.1.1.10 255.255.255.0

interface GigabitEthernet2

nameif OUTSIDE1

security-level 98

ip address 20.1.1.10 255.255.255.0

interface GigabitEthernet3

nameif OUTSIDE2

security-level 97

ip address 40.1.1.10 255.255.255.0

access-list OUT1-IN extended permit udp 20.1.1.0 255.255.255.0 10.1.1.0 255.255.255.0 eq 848

access-list OUT1-IN extended permit udp 20.1.1.0 255.255.255.0 30.1.1.0 255.255.255.0 eq 848

access-list OUT2-IN extended permit udp 40.1.1.0 255.255.255.0 10.1.1.0 255.255.255.0 eq 848

access-list OUT2-IN extended permit udp 40.1.1.0 255.255.255.0 20.1.1.0 255.255.255.0 eq 848

access-list IN2-IN1 extended permit udp 30.1.1.0 255.255.255.0 10.1.1.0 255.255.255.0 eq 848

access-list IN2-IN1 extended permit esp host 3.3.3.3 host 4.4.4.4

access-list OUT2-IN extended permit esp host 4.4.4.4 host 3.3.3.3

access-group IN2-IN1 in interface INSIDE2

access-group OUT1-IN in interface OUTSIDE1

access-group OUT2-IN in interface OUTSIDE2

route INSIDE2 3.3.3.3 255.255.255.255 30.1.1.1 1

route OUTSIDE2 4.4.4.4 255.255.255.255 40.1.1.1 1

KEYSERVER

R1 (KS1)

crypto isakmp policy 10

authentication pre-share

crypto isakmp key cisco address 0.0.0.0 0.0.0.0

crypto ipsec transform-set GSET esp-des esp-sha-hmac

crypto ipsec profile GET

set transform-set GSET

crypto gdoi group CCIE

identity number 123

server local

rekey retransmit 10 number 2

rekey authentication mypubkey rsa KEK

rekey transport unicast

sa ipsec 1

profile GET

match address ipv4 101

replay time window-size 50

address ipv4 10.1.1.1

redundancy

local priority 100

peer address ipv4 20.1.1.1

interface FastEthernet0/0

ip address 10.1.1.1 255.255.255.0

ip route 20.1.1.0 255.255.255.0 10.1.1.10

ip route 30.1.1.0 255.255.255.0 10.1.1.10

ip route 40.1.1.0 255.255.255.0 10.1.1.10

access-list 101 permit ip host 3.3.3.3 host 4.4.4.4

access-list 101 permit ip host 4.4.4.4 host 3.3.3.3

R2(KS2)

crypto isakmp policy 10

authentication pre-share

crypto isakmp key cisco address 0.0.0.0 0.0.0.0

crypto ipsec transform-set GSET esp-des esp-sha-hmac

crypto ipsec profile GET

set transform-set GSET

crypto gdoi group CCIE

identity number 123

server local

rekey retransmit 10 number 2

rekey authentication mypubkey rsa KEK

rekey transport unicast

sa ipsec 1

profile GET

match address ipv4 101

replay time window-size 50

address ipv4 20.1.1.1

redundancy

local priority 50

peer address ipv4 10.1.1.1

interface FastEthernet0/0

ip address 20.1.1.1 255.255.255.0

ip route 10.1.1.0 255.255.255.0 20.1.1.10

ip route 30.1.1.0 255.255.255.0 20.1.1.10

ip route 40.1.1.0 255.255.255.0 20.1.1.10

access-list 101 permit ip host 3.3.3.3 host 4.4.4.4

access-list 101 permit ip host 4.4.4.4 host 3.3.3.3

R3(GM-1)

crypto isakmp policy 10

authentication pre-share

crypto isakmp key cisco address 10.1.1.1

crypto isakmp key cisco address 20.1.1.1

crypto gdoi group CCIE

identity number 123

server address ipv4 10.1.1.1

server address ipv4 20.1.1.1

crypto map CMAP 10 gdoi

set group CCIE

interface Loopback0

ip address 3.3.3.3 255.255.255.255

interface FastEthernet0/0

ip address 30.1.1.1 255.255.255.0

crypto map CMAP

ip route 4.4.4.4 255.255.255.255 30.1.1.10

ip route 10.1.1.0 255.255.255.0 30.1.1.10

ip route 20.1.1.0 255.255.255.0 30.1.1.10

ip route 40.1.1.0 255.255.255.0 30.1.1.10

R4(GM-2)

crypto isakmp policy 10

authentication pre-share

crypto isakmp key cisco address 20.1.1.1

crypto isakmp key cisco address 10.1.1.1

crypto gdoi group CCIE

identity number 123

server address ipv4 20.1.1.1

server address ipv4 10.1.1.1

crypto map CMAP 10 gdoi

set group CCIE

interface Loopback0

ip address 4.4.4.4 255.255.255.255

interface FastEthernet0/0

ip address 40.1.1.1 255.255.255.0

crypto map CMAP

ip route 3.3.3.3 255.255.255.255 40.1.1.10

ip route 10.1.1.0 255.255.255.0 40.1.1.10

ip route 20.1.1.0 255.255.255.0 40.1.1.10

ip route 30.1.1.0 255.255.255.0 40.1.1.10

!