EASY VPN TERMINATED ON ASA SERVER WITH HARDWARE & SOFTWARE CLIENT

ASSIGNMENT - 15

CLIENT

int lo0

ip add 2.2.2.2 255.255.255.255

int fa0/0

duplex full

ip add 20.1.1.1 255.255.255.0

no shut

ip route 50.1.1.0 255.255.255.0 20.1.1.10

ip route 10.1.1.0 255.255.255.0 20.1.1.10

ip route 30.1.1.0 255.255.255.0 20.1.1.10

ip route 40.1.1.0 255.255.255.0 20.1.1.10

ip route 1.1.1.1 255.255.255.255 20.1.1.10

crypto isakmp policy 10

authentication pre-share

encryption 3des

group 2

hash md5

exit

crypto ipsec client ezvpn CLIENT

group EASY key cisco123

peer 10.1.1.1

mode client

connect auto

exit

int lo0

crypto ipsec client ezvpn CLIENT inside

int fa0/0

crypto ipsec client ezvpn CLIENT outside

NEM

int lo0

ip add 3.3.3.3 255.255.255.255

int fa0/0

ip add 30.1.1.1 255.255.255.0

no shut

ip route 50.1.1.0 255.255.255.0 30.1.1.10

ip route 10.1.1.0 255.255.255.0 30.1.1.10

ip route 30.1.1.0 255.255.255.0 30.1.1.10

ip route 40.1.1.0 255.255.255.0 30.1.1.10

ip route 1.1.1.1 255.255.255.255 30.1.1.10

crypto isakmp policy 10

authentication pre-share

encryption 3des

group 2

hash md5

exit

crypto ipsec client ezvpn NEM

group EASY key cisco123

peer 10.1.1.1

mode network-extension

connect manual

exit

int lo0

crypto ipsec client ezvpn NEM inside

int fa0/0

crypto ipsec client ezvpn NEM outside

exit

crypto ipsec client ezvpn connect

NEM_PLUS

int lo0

ip add 4.4.4.4 255.255.255.255

int fa0/0

ip add 40.1.1.1 255.255.255.0

no shut

ip route 50.1.1.0 255.255.255.0 40.1.1.10

ip route 10.1.1.0 255.255.255.0 40.1.1.10

ip route 30.1.1.0 255.255.255.0 40.1.1.10

ip route 40.1.1.0 255.255.255.0 40.1.1.10

ip route 1.1.1.1 255.255.255.255 40.1.1.10

crypto isakmp policy 10

authentication pre-share

encryption 3des

group 2

hash md5

exit

crypto ipsec client ezvpn NEM-PLUS

group EASY key cisco123

peer 10.1.1.1

mode network-plus

connect auto

exit

int lo0

crypto ipsec client ezvpn NEM-PLUS inside

int fa0/0

crypto ipsec client ezvpn NEM-PLUS outside

exit

INTERNET

int fa0/0

duplex full

ip add 10.1.1.10 255.255.255.0

no shut

int fa0/1

duplex full

ip add 20.1.1.10 255.255.255.0

no shut

int fa1/0

duplex full

ip add 30.1.1.10 255.255.255.0

no shut

int fa1/1

duplex full

ip add 40.1.1.10 255.255.255.0

no shut

int fa2/0

duplex full

ip add 60.1.1.10 255.255.255.0

no shut

ip route 50.1.1.0 255.255.255.0 10.1.1.1

ASA_SERVER

int e0/0

nameif INSIDE

ip add 50.1.1.1 255.255.255.0

no shut

int e0/1

nameif OUTSIDE

ip add 10.1.1.1 255.255.255.0

no shut

route OUTSIDE 20.1.1.0 255.255.255.0 10.1.1.10

route OUTSIDE 30.1.1.0 255.255.255.0 10.1.1.10

route OUTSIDE 40.1.1.0 255.255.255.0 10.1.1.10

route OUTSIDE 2.2.2.2 255.255.255.255 10.1.1.10

route OUTSIDE 3.3.3.3 255.255.255.255 10.1.1.10

route OUTSIDE 4.4.4.4 255.255.255.255 10.1.1.10

route INSIDE 1.1.1.1 255.255.255.255 50.1.1.2

route OUTSIDE 60.1.1.0 255.255.255.0 10.1.1.10

access-list OUT-IN permit icmp any any

access-group OUT-IN in interface OUTSIDE

EASY VPN CONFIGURATION

crypto isakmp policy 10

authentication pre-share

encryption 3des

hash md5

group 2

lifetime 86400

tunnel-group EASY type remote-access

tunnel-group EASY ipsec-attributes

pre-shared-key cisco123

username cisco password cisco

ip local pool EASY-POOL 7.7.7.1-7.7.7.5

access-list SPLIT-ACL extended permit ip host 1.1.1.1 any

group-policy EASY-POLICY internal

group-policy EASY-POLICY attributes

address-pools value EASY-POOL

split-tunnel-policy tunnelspecified

split-tunnel-network-list value SPLIT-ACL

nem enable

crypto ipsec transform-set TSET esp-3des esp-md5-hmac

crypto dynamic-map DMAP 100 set transform-set TSET

crypto dynamic-map DMAP 100 set reverse-route

crypto map SMAP 100 ipsec-isakmp dynamic DMAP

tunnel-group EASY general-attributes

default-group-policy EASY-POLICY

crypto map SMAP interface OUTSIDE

crypto isakmp enable OUTSIDE

Lo0

int fa0/0

ip add 50.1.1.2 255.255.255.0

no shut

int lo0

ip add 1.1.1.1 255.255.255.0

ip route 20.1.1.0 255.255.255.0 50.1.1.1

ip route 10.1.1.0 255.255.255.0 50.1.1.1

ip route 30.1.1.0 255.255.255.0 50.1.1.1

ip route 40.1.1.0 255.255.255.0 50.1.1.1

ip route 7.7.7.0 255.255.255.0 50.1.1.1

ip route 2.2.2.2 255.255.255.255 50.1.1.1

ip route 3.3.3.3 255.255.255.255 50.1.1.1

ip route 4.4.4.4 255.255.255.255 50.1.1.1

ip route 60.1.1.0 255.255.255.0 50.1.1.1