EASY VPN USING DVTI

ASSIGNMENT - 16

INTERNET

interface FastEthernet0/0

ip address 20.1.1.10 255.255.255.0

interface FastEthernet0/1

ip address 10.1.1.10 255.255.255.0

R1

crypto isakmp policy 10

encr 3des

hash md5

authentication pre-share

group 2

crypto isakmp client configuration group CCIE

key cisco123

pool POOL

crypto isakmp profile ISA_PRO

match identity group CCIE

client authentication list AAA

isakmp authorization list AUTH

client configuration address respond

virtual-template 1

crypto ipsec transform-set ESET esp-3des esp-sha-hmac

crypto ipsec profile IP_PRO

set transform-set ESET

set isakmp-profile ISA_PRO

interface Loopback0

ip address 1.1.1.1 255.255.255.255

interface FastEthernet0/0

ip address 10.1.1.1 255.255.255.0

interface Virtual-Template1 type tunnel

ip unnumbered FastEthernet0/0

tunnel source FastEthernet0/0

tunnel mode ipsec ipv4

tunnel protection ipsec profile IP_PRO

router eigrp 100

network 1.1.1.1 0.0.0.0

network 10.1.1.0 0.0.0.255

no auto-summary

ip local pool POOL 7.7.7.1 7.7.7.10

ip forward-protocol nd

ip route 20.1.1.0 255.255.255.0 10.1.1.10

R3

crypto ipsec client ezvpn CLIENT

connect auto

group CCIE key cisco123

mode client

peer 10.1.1.1

virtual-interface 1

xauth userid mode interactive

interface Loopback0

ip address 3.3.3.3 255.255.255.255

crypto ipsec client ezvpn CLIENT inside

interface FastEthernet0/0

ip address 20.1.1.1 255.255.255.0

crypto ipsec client ezvpn CLIENT

interface Virtual-Template1 type tunnel

tunnel mode ipsec ipv4

router eigrp 100

network 2.2.2.2 0.0.0.0

network 7.7.7.0 0.0.0.255

no auto-summary

ip route 10.1.1.0 255.255.255.0 20.1.1.10