EASY VPN USING ROUTERS

ASSIGNMENT 17

TASK:

SERVER

- Configure R1 as easy-vpn server with 2 groups and a single pool

- The pool of IP address are from the range 20.1.1.2 to 20.1.1.100

- The first group name is HR and PSK is also HR

- The second group name is Admin and PSK is also Admin

- Create two usernames namely CCIE & CCNP with password as CCIE and CCNAP respectively

- Group HR should be able to access both 1.1.1.1 & 10.1.1.1

CLIENT

- Configure R2 as EASY VPN client operating in client mode for group HR & to connect manually such that all the internal traffic of R2 is encrypted ( 2 inside statement & 1 outside statement)

- Configure R3 as EASY VPN client in NEM mode for group ADMIN with a connection method as auto and allow that client to save the password in the local database, all the traffic flown from the internal network of R3 should be encrypted while reaching the internal network of R1

SOLUTION:

R1

SERVER

aaa new-model

aaa authentication login AAA local

aaa authorization network AUTH local

username CCIE password 0 CCIE

username CCNP password 0 CCNP

crypto isakmp policy 10

encr 3des

hash md5

authentication pre-share

group 2

crypto isakmp client configuration group HR

key HR

pool POOL

acl 101

crypto isakmp client configuration group Admin

key Admin

pool POOL

acl 102

save-password

crypto ipsec transform-set ESET esp-3des esp-md5-hmac

crypto dynamic-map DMAP 10

set transform-set ESET

reverse-route

crypto map CMAP client authentication list AAA

crypto map CMAP isakmp authorization list AUTH

crypto map CMAP client configuration address respond

crypto map CMAP 10 ipsec-isakmp dynamic DMAP

interface FastEthernet0/0

crypto map CMAP

ip local pool POOL 20.1.1.2 20.1.1.100

ip route 2.2.2.2 255.255.255.255 10.1.14.10

ip route 3.3.3.3 255.255.255.255 10.1.14.10

ip route 10.1.234.0 255.255.255.0 10.1.14.10

ip route 22.22.22.22 255.255.255.255 10.1.14.10

ip route 33.33.33.33 255.255.255.255 10.1.14.10

access-list 101 permit ip host 1.1.1.1 any

access-list 102 permit ip host 1.1.1.1 any

access-list 102 permit ip host 10.1.1.1 any

R2

CLIENT 1

int lo0

ip add 2.2.2.2 255.255.255.255

int fa0/0

duplex full

ip add 20.1.1.1 255.255.255.0

no shut

ip route 50.1.1.0 255.255.255.0 20.1.1.10

ip route 10.1.1.0 255.255.255.0 20.1.1.10

ip route 30.1.1.0 255.255.255.0 20.1.1.10

ip route 40.1.1.0 255.255.255.0 20.1.1.10

ip route 1.1.1.1 255.255.255.255 20.1.1.10

crypto isakmp policy 10

authentication pre-share

encryption 3des

group 2

hash md5

exit

crypto ipsec client ezvpn CLIENT

group HR key HR

peer 10.1.14.1

mode client

connect manual

exit

int lo0

crypto ipsec client ezvpn CLIENT inside

int lo1

crypto ipsec client ezvpn CLIENT inside

int fa0/0

crypto ipsec client ezvpn CLIENT outside

exit

crypto ipsec client ezvpn connect

ip route 1.1.1.1 255.255.255.255 10.1.234.10

ip route 10.1.1.1 255.255.255.255 10.1.234.10

ip route 10.1.14.0 255.255.255.0 10.1.234.10

R3

NEM

int lo0

ip add 3.3.3.3 255.255.255.255

int fa0/0

ip add 30.1.1.1 255.255.255.0

no shut

crypto isakmp policy 10

authentication pre-share

encryption 3des

group 2

hash md5

exit

crypto ipsec client ezvpn NEM

group Admin key Admin

peer 10.1.14.1

mode network-extension

connect auto

exit

int lo0

crypto ipsec client ezvpn NEM inside

int lo1

crypto ipsec client ezvpn NEM inside

int fa0/0

crypto ipsec client ezvpn NEM outside

exit

ip route 1.1.1.1 255.255.255.255 10.1.234.10

ip route 10.1.1.1 255.255.255.255 10.1.234.10

ip route 10.1.14.0 255.255.255.0 10.1.234.10

INTERNET

int Fa0/0

ip address 10.1.234.10 255.255.255.0

int Fa0/1

ip address 10.1.14.10 255.255.255.0