SSL VPN

ASSIGNMENT 18

R1

username cisco privilege 15 password 0 cisco

interface FastEthernet0/0

ip address 10.1.1.1 255.255.255.0

ip route 20.1.1.0 255.255.255.0 10.1.1.10

ip http server

ip http authentication local

SSL-GATEWAY

crypto pki trustpoint TP-self-signed-4279256517

enrollment selfsigned

subject-name cn=IOS-Self-Signed-Certificate-4279256517

revocation-check none

rsakeypair TP-self-signed-4279256517

crypto pki certificate chain TP-self-signed-4279256517

certificate self-signed 01 nvram:IOS-Self-Sig#1.cer

username DOLIA@CCIE password 0 DOLIA

username VARUN@CCNP password 0 VARUN

interface FastEthernet0/0

ip address 20.1.1.10 255.255.255.0

interface FastEthernet0/1

ip address 10.1.1.10 255.255.255.0

ip http server

webvpn gateway SSL-GW

hostname GATEWAY

ip address 20.1.1.10 port 8080

ssl trustpoint TP-self-signed-4279256517

logging enable

inservice

webvpn context CCIE

ssl authenticate verify all

url-list "R1"

url-text "R1" url-value "10.1.1.1"

login-message "WELCOME"

port-forward "TRAINER-PORT"

local-port 2300 remote-server "10.1.1.1" remote-port 23 description "TELNET PORT FORWARDING"

local-port 8000 remote-server "10.1.1.1" remote-port 80 description "HTTP PORT FORWARDING"

policy group TRAINER-POLICY

url-list "R1"

port-forward "TRAINER-PORT"

functions file-access

functions file-browse

functions file-entry

functions svc-enabled

banner "WELCOME CCIE"

default-group-policy TRAINER-POLICY

aaa authentication list AAA

aaa authentication domain @CCIE

gateway SSL-GW domain CCIE

inservice

webvpn context CCNP

ssl authenticate verify all

login-message "WELCOME"

policy group STUDENT-POLICY

banner "WELCOME STUDENT"

hide-url-bar

default-group-policy STUDENT-POLICY

aaa authentication list AAA

aaa authentication domain @CCNP

gateway SSL-GW domain CCNP

inservice