IPSEC STATEFULL FAILOVER USING HSRP & SSO

ASSIGNMENT 19

R1

int fa0/0

ip add 10.1.1.1 255.255.255.0

no shut

int lo0

ip add 1.1.1.1 255.255.255.255

ip route 10.1.2.0 255.255.255.0 10.1.1.100

ip route 20.1.2.0 255.255.255.0 10.1.1.100

ip route 20.1.1.0 255.255.255.0 10.1.1.100

ip route 2.2.2.2 255.255.255.255 10.1.1.100

R2

int fa0/0

ip add 20.1.1.1 255.255.255.0

no shut

int lo0

ip add 2.2.2.2 255.255.255.255

ip route 20.1.2.0 255.255.255.0 20.1.1.100

ip route 10.1.2.0 255.255.255.0 20.1.1.100

ip route 10.1.1.0 255.255.255.0 20.1.1.100

ip route 1.1.1.1 255.255.255.255 20.1.1.100

R3

int fa0/0

standby 1 ip 10.1.2.100

standby 1 priority 100

standby 1 preempt

standby 1 name STATEFULL-FAILOVER

standby 1 track fa0/1

int fa0/1

standby 2 ip 10.1.1.100

standby 2 priority 100

standby 2 preempt

standby 2 track fa0/0

access-list 101 permit ip host 1.1.1.1 host 2.2.2.2

crypto isakmp policy 10

encr aes 256

hash sha

authentication pre-share

group 2

lifetime 86400

crypto isakmp key cisco123 address 20.1.2.100

crypto ipsec transform-set TSET esp-3des esp-md5-hmac

mode tunnel

crypto map CMAP 10 ipsec-isakmp

set peer 20.1.2.100

set transform-set TSET

match address 101

int fa0/0

crypto map CMAP redundancy STATEFULL-FAILOVER stateful

redundancy inter-device

scheme standby STATEFULL-FAILOVER

exit

ipc zone default

association 1

protocol sctp

local-port 12345

local-ip 10.1.2.2

exit

remote-port 12345

remote-ip 10.1.2.3

exit

exit

exit

exit

ip route 20.1.2.0 255.255.255.0 10.1.2.1

ip route 20.1.1.0 255.255.255.0 10.1.2.1

ip route 2.2.2.2 255.255.255.255 10.1.2.1

ip route 1.1.1.1 255.255.255.255 10.1.1.1

R4

int fa0/0

standby 1 ip 10.1.2.100

standby 1 priority 50

standby 1 preempt

standby 1 name STATEFULL-FAILOVER

standby 1 track fa0/1

int fa0/1

standby 2 ip 10.1.1.100

standby 2 priority 50

standby 2 preempt

standby 2 track fa0/0

access-list 101 permit ip host 1.1.1.1 host 2.2.2.2

crypto isakmp policy 10

encr aes 256

hash sha

authentication pre-share

group 2

lifetime 86400

crypto isakmp key cisco123 address 20.1.2.100

crypto ipsec transform-set TSET esp-3des esp-md5-hmac

mode tunnel

crypto map CMAP 10 ipsec-isakmp

set peer 20.1.2.100

set transform-set TSET

match address 101

int fa0/0

crypto map CMAP redundancy STATEFULL-FAILOVER stateful

redundancy inter-device

scheme standby STATEFULL-FAILOVER

exit

ipc zone default

association 1

protocol sctp

local-port 12345

local-ip 10.1.2.3

exit

remote-port 12345

remote-ip 10.1.2.2

exit

exit

exit

exit

ip route 20.1.2.0 255.255.255.0 10.1.2.1

ip route 20.1.1.0 255.255.255.0 10.1.2.1

ip route 2.2.2.2 255.255.255.255 10.1.2.1

ip route 1.1.1.1 255.255.255.255 10.1.1.1

R5

int fa0/0

standby 1 ip 20.1.2.100

standby 1 priority 100

standby 1 preempt

standby 1 name STATEFULL-FAILOVER

standby 1 track fa0/1

int fa0/1

standby 2 ip 20.1.1.100

standby 2 priority 100

standby 2 preempt

standby 2 track fa0/0

access-list 101 permit ip host 2.2.2.2 host 1.1.1.1

crypto isakmp policy 10

encr aes 256

hash sha

authentication pre-share

group 2

lifetime 86400

crypto isakmp key cisco123 address 10.1.2.100

crypto ipsec transform-set TSET esp-3des esp-md5-hmac

mode tunnel

crypto map CMAP 10 ipsec-isakmp

set peer 10.1.2.100

set transform-set TSET

match address 101

int fa0/0

crypto map CMAP redundancy STATEFULL-FAILOVER stateful

redundancy inter-device

scheme standby STATEFULL-FAILOVER

exit

ipc zone default

association 1

protocol sctp

local-port 12345

local-ip 20.1.2.2

exit

remote-port 12345

remote-ip 20.1.2.3

exit

exit

exit

exit

ip route 10.1.2.0 255.255.255.0 20.1.2.1

ip route 10.1.1.0 255.255.255.0 20.1.2.1

ip route 1.1.1.1 255.255.255.255 20.1.2.1

ip route 2.2.2.2 255.255.255.255 20.1.1.1

R6

int fa0/0

standby 1 ip 20.1.2.100

standby 1 priority 50

standby 1 preempt

standby 1 name STATEFULL-FAILOVER

standby 1 track fa0/1

int fa0/1

standby 2 ip 20.1.1.100

standby 2 priority 50

standby 2 preempt

standby 2 track fa0/0

access-list 101 permit ip host 2.2.2.2 host 1.1.1.1

crypto isakmp policy 10

encr aes 256

hash sha

authentication pre-share

group 2

lifetime 86400

crypto isakmp key cisco123 address 10.1.2.100

crypto ipsec transform-set TSET esp-3des esp-md5-hmac

mode tunnel

crypto map CMAP 10 ipsec-isakmp

set peer 10.1.2.100

set transform-set TSET

match address 101

int fa0/0

crypto map CMAP redundancy STATEFULL-FAILOVER stateful

redundancy inter-device

scheme standby STATEFULL-FAILOVER

exit

ipc zone default

association 1

protocol sctp

local-port 12345

local-ip 20.1.2.3

exit

remote-port 12345

remote-ip 20.1.2.2

exit

exit

exit

exit

ip route 10.1.2.0 255.255.255.0 20.1.2.1

ip route 10.1.1.0 255.255.255.0 20.1.2.1

ip route 1.1.1.1 255.255.255.255 20.1.2.1

ip route 2.2.2.2 255.255.255.255 20.1.1.1

Internet

int fa0/0

ip add 10.1.2.1 255.255.255.0

no shut

int fa0/1

ip add 20.1.2.1 255.255.255.0

no shut

ip route 10.1.1.0 255.255.255.0 10.1.2.100

ip route 20.1.1.0 255.255.255.0 20.1.2.100