IPSEC SITE TO SITE VPN IN TUNNEL MODE PASSING ASA WITH NAT ENABLED

WITH SAME SECURITY ZONES

ASSIGNMENT - 2

R1

crypto isakmp policy 10

encr aes 256

authentication pre-share

group 2

crypto isakmp key cisco address 20.1.1.1

crypto ipsec transform-set R1R2-TSET esp-3des esp-md5-hmac

mode tunnel

crypto map R1R2-CMAP 10 ipsec-isakmp

set peer 20.1.1.1

set transform-set R1R2-TSET

match address 101

interface loopback0

ip address 1.1.1.1 255.255.255.0

interface FastEthernet0/0

ip address 10.1.1.1 255.255.255.0

crypto map R1R2-CMAP

ip route 20.1.1.0 255.255.255.0 10.1.1.10

access-list 101 permit ip host 1.1.1.1 host 2.2.2.2

ASA

interface Ethernet0/0

nameif INSIDE1

security-level 100

ip address 10.1.1.10 255.255.255.0

interface Ethernet0/1

nameif INSIDE2

security-level 100

ip address 20.1.1.10 255.255.255.0

static (INSIDE1,INSIDE2) 4.4.4.4 10.1.1.1 netmask 255.255.255.255

same-security traffic permit inter-interface

R2

crypto isakmp policy 10

encr aes 256

authentication pre-share

group 2

crypto isakmp key cisco address 4.4.4.4

crypto ipsec transform-set R1R2-TSET esp-3des esp-md5-hmac

mode tunnel

crypto map R1R2-CMAP 10 ipsec-isakmp

set peer 4.4.4.4

set transform-set R1R2-TSET

match address 101

interface FastEthernet0/0

ip address 20.1.1.1 255.255.255.0

crypto map R1R2-CMAP

ip route 4.4.4.4 255.255.255.255 20.1.1.10

access-list 101 permit ip host 2.2.2.2 host 4.4.4.4