DUAL HUB SINGLE CLOUD DMVPN USING EIGRP

ASSIGNMENT 20

PHASE 1 (ALL TRAFFIC MUST GO VIA HUB)

HUB1

crypto isakmp policy 10

authentication pre-share

crypto isakmp key cisco address 0.0.0.0 0.0.0.0

crypto ipsec transform-set TSET esp-3des esp-sha-hmac

crypto ipsec profile DMVPN

set transform-set TSET

interface Loopback0

ip address 1.1.1.1 255.255.255.255

interface Tunnel0

ip address 172.16.1.1 255.255.255.0

no ip redirects

ip nhrp authentication cisco123

ip nhrp map multicast dynamic

ip nhrp network-id 100

no ip split-horizon eigrp 100

tunnel source FastEthernet0/0

tunnel mode gre multipoint

tunnel key 123

tunnel protection ipsec profile DMVPN

interface FastEthernet0/0

ip address 10.1.1.1 255.255.255.0

router eigrp 100

network 1.1.1.1 0.0.0.0

network 172.16.1.0 0.0.0.255

no auto-summary

ip route 30.1.1.0 255.255.255.0 10.1.1.10

ip route 40.1.1.0 255.255.255.0 10.1.1.10

HUB2

crypto isakmp policy 10

authentication pre-share

crypto isakmp key cisco address 0.0.0.0 0.0.0.0

crypto ipsec transform-set TSET esp-3des esp-sha-hmac

crypto ipsec profile DMVPN

set transform-set TSET

interface Loopback0

ip address 2.2.2.2 255.255.255.255

interface Tunnel0

ip address 172.16.1.2 255.255.255.0

no ip redirects

ip nhrp authentication cisco123

ip nhrp map multicast dynamic

ip nhrp network-id 100

no ip split-horizon eigrp 100

tunnel source FastEthernet0/0

tunnel mode gre multipoint

tunnel key 123

tunnel protection ipsec profile DMVPN

interface FastEthernet0/0

ip address 20.1.1.1 255.255.255.0

router eigrp 100

network 2.2.2.2 0.0.0.0

network 172.16.1.0 0.0.0.255

no auto-summary

ip route 30.1.1.0 255.255.255.0 20.1.1.10

ip route 40.1.1.0 255.255.255.0 20.1.1.10

SPOKE 1

crypto isakmp policy 10

authentication pre-share

crypto isakmp key cisco address 0.0.0.0 0.0.0.0

crypto ipsec transform-set TSET esp-3des esp-sha-hmac

crypto ipsec profile DMVPN

set transform-set TSET

interface Loopback0

ip address 3.3.3.3 255.255.255.255

interface Tunnel0

ip address 172.16.1.3 255.255.255.0

no ip redirects

ip nhrp authentication cisco123

ip nhrp map 172.16.1.1 10.1.1.1

ip nhrp map 172.16.1.2 20.1.1.1

ip nhrp map multicast 10.1.1.1

ip nhrp map multicast 20.1.1.1

ip nhrp network-id 100

ip nhrp nhs 172.16.1.1

ip nhrp nhs 172.16.1.2

tunnel source FastEthernet0/0

tunnel mode gre multipoint

tunnel key 123

tunnel protection ipsec profile DMVPN

interface FastEthernet0/0

ip address 30.1.1.1 255.255.255.0

router eigrp 100

network 3.3.3.3 0.0.0.0

network 172.16.1.0 0.0.0.255

distance 100 172.16.1.2 0.0.0.0 99

no auto-summary

ip route 10.1.1.0 255.255.255.0 30.1.1.10

ip route 20.1.1.0 255.255.255.0 30.1.1.10

ip route 40.1.1.0 255.255.255.0 30.1.1.10

access-list 99 permit 2.2.2.2

access-list 99 permit 4.4.4.4

SPOKE 2

crypto isakmp policy 10

authentication pre-share

crypto isakmp key cisco address 0.0.0.0 0.0.0.0

crypto ipsec transform-set TSET esp-3des esp-sha-hmac

crypto ipsec profile DMVPN

set transform-set TSET

interface Loopback0

ip address 4.4.4.4 255.255.255.255

interface Tunnel0

ip address 172.16.1.4 255.255.255.0

no ip redirects

ip nhrp authentication cisco123

ip nhrp map 172.16.1.1 10.1.1.1

ip nhrp map 172.16.1.2 20.1.1.1

ip nhrp map multicast 10.1.1.1

ip nhrp map multicast 20.1.1.1

ip nhrp network-id 100

ip nhrp nhs 172.16.1.1

ip nhrp nhs 172.16.1.2

tunnel source FastEthernet0/0

tunnel mode gre multipoint

tunnel key 123

tunnel protection ipsec profile DMVPN

interface FastEthernet0/0

ip address 40.1.1.1 255.255.255.0

router eigrp 100

network 4.4.4.4 0.0.0.0

network 172.16.1.0 0.0.0.255

distance 100 172.16.1.2 0.0.0.0 99

no auto-summary

ip route 10.1.1.0 255.255.255.0 40.1.1.10

ip route 20.1.1.0 255.255.255.0 40.1.1.10

ip route 30.1.1.0 255.255.255.0 40.1.1.10

access-list 99 permit 2.2.2.2

access-list 99 permit 3.3.3.3

INTERNET

interface FastEthernet0/0

ip address 10.1.1.10 255.255.255.0

interface FastEthernet0/1

ip address 20.1.1.10 255.255.255.0

interface FastEthernet1/0

ip address 30.1.1.10 255.255.255.0

interface FastEthernet1/1

ip address 40.1.1.10 255.255.255.0

PHASE 2 (ALL TRAFFIC MUST GO DIRECTLY)

HUB1

interface tunnel 0

no ip next-hop-self eigrp 100

HUB2

interface tunnel 0

no ip next-hop-self eigrp 100

PHASE 3 (ALL TRAFFIC MUST GO DIRECTLY but with CEF valid and scalability)

HUB1

interface tunnel 0

ip next-hop-self eigrp 100

ip nhrp redirect

HUB2

interface tunnel 0

ip next-hop-self eigrp 100

ip nhrp redirect

SPOKE1

interface tunnel 0

ip nhrp shortcut

SPOKE2

interface tunnel 0

ip nhrp shortcut