IPSEC SITE TO SITE VPN IN TRANSPORT MODE PASSING ASA WITH NAT ENABLED

WITH DIFFERENT SECURITY ZONES

ASSIGNMENT - 3

R1

crypto isakmp policy 10

encr aes 256

authentication pre-share

group 2

crypto isakmp key cisco address 2.2.2.1

crypto ipsec transform-set R1R2-TSET esp-3des esp-md5-hmac

mode transport

crypto map R1R2-CMAP 10 ipsec-isakmp

set peer 2.2.2.1

set transform-set R1R2-TSET

match address 101

interface FastEthernet0/0

ip address 1.1.1.1 255.255.255.0

crypto map R1R2-CMAP

ip route 2.2.2.0 255.255.255.0 1.1.1.10

access-list 101 permit ip host 1.1.1.1 host 2.2.2.1

ASA

interface Ethernet0/0

nameif INSIDE

security-level 100

ip address 1.1.1.10 255.255.255.0

interface Ethernet0/1

nameif OUTSIDE

security-level 0

ip address 2.2.2.10 255.255.255.0

static (INSIDE1,INSIDE2) 4.4.4.4 1.1.1.1 netmask 255.255.255.255

access-list OUT-IN permit udp host 2.2.2.1 host 4.4.4.4 eq 500

access-list OUT-IN permit udp host 2.2.2.1 host 4.4.4.4 eq 4500

access-group OUT-IN in interface OUTSIDE

R2

crypto isakmp policy 10

encr aes 256

authentication pre-share

group 2

crypto isakmp key cisco address 4.4.4.4

crypto ipsec transform-set R1R2-TSET esp-3des esp-md5-hmac

mode transport

crypto map R1R2-CMAP 10 ipsec-isakmp

set peer 4.4.4.4

set transform-set R1R2-TSET

match address 101

interface FastEthernet0/0

ip address 2.2.2.1 255.255.255.0

crypto map R1R2-CMAP

ip route 1.1.1.1 255.255.255.255 2.2.2.10

ip route 4.4.4.4 255.255.255.255 2.2.2.10

access-list 101 permit ip host 2.2.2.1 host 4.4.4.4

Note : Phase 1 tunnel will form by default for traffic initiated from INSIDE to OUTSIDE (R1 to R2) as ASA inspects all UDP traffic from higher security level to a lower security level. But if the traffic is initiated from OUTSIDE to INSIDE (R2 to R1) then ASA denies UDP 500. In that case we create an access-list in ASA.

access-list OUT-IN permit udp host 2.2.2.1 host 4.4.4.4 eq 500

Phase 2 tunnel will also form by default for traffic initiated from INSIDE to OUTSIDE (R1 to R2) as ASA inspects all UDP traffic from higher security level to a lower security level. But if the traffic is initiated from OUTSIDE to INSIDE (R2 to R1) then ASA denies UDP 4500 (and not ESP as ESP will be encapsulated inside UDP 4500 when NAT is enabled in ASA. In that case we create an access-list in ASA.

access-list OUT-IN permit udp host 2.2.2.1 host 4.4.4.4 eq 4500

When NAT is not enabled and when traffic is initiated from OUTSIDE to INSIDE then the below access-list is given in the ASA as the encrypted packets will be encapsulated in ESP.

access-list OUT-IN permit esp host 2.2.2.1 host 4.4.4.4

Since NAT is enabled in ASA R2’s peer address,R2’s key address R2’s static routes, R2’s crypto ACL’s & ASA’s access-lists all must point to the translated address only

We need to create two static routes in R2 one for reaching the 1.1.1.0/24 (real) network and the other for reaching the translated 4.4.4.4/32 (global) ip address of the real ip 1.1.1.1/32

ip route 1.1.1.1 255.255.255.255 2.2.2.10

ip route 4.4.4.4 255.255.255.255 2.2.2.10