IPSEC SITE TO SITE VPN TERMINATED ON ASA 8.0 & 8.2

ASSIGNMENT - 4

Interesting traffic is between R1’s 10.1.1.1 to R2’s 2.2.2.1

R1

interface loopback 0

ip address 10.1.1.1 255.255.255.0

interface FastEthernet0/0

ip address 1.1.1.1 255.255.255.0

ip route 2.2.2.0 255.255.255.0 1.1.1.10

ASA

interface Ethernet0/0

nameif INSIDE

security-level 100

ip address 1.1.1.10 255.255.255.0

interface Ethernet0/1

nameif OUTSIDE

security-level 0

ip address 2.2.2.10 255.255.255.0

crypto ipsec transform-set TSET esp-3des esp-sha-hmac

crypto map CMAP 10 ipsec-isakmp

crypto map CMAP 10 match address 101

crypto map CMAP 10 set peer 2.2.2.1

crypto map CMAP 10 set transform-set TSET

crypto isakmp enable OUTSIDE

crypto isakmp policy 10

authentication pre-share

encryption 3des

hash sha

group 2

lifetime 86400

route INSIDE 10.1.1.1 255.255.255.0 1.1.1.1

access-list 101 permit ip host 10.1.1.1 host 2.2.2.1

R2

crypto isakmp policy 10

encr aes 256

authentication pre-share

group 2

crypto isakmp key cisco address 4.4.4.4

crypto ipsec transform-set R1R2-TSET esp-3des esp-md5-hmac

crypto map R1R2-CMAP 10 ipsec-isakmp

set peer 2.2.2.10

set transform-set R1R2-TSET

match address 101

interface FastEthernet0/0

ip address 2.2.2.1 255.255.255.0

crypto map R1R2-CMAP

ip route 10.1.1.1 255.255.255.255 2.2.2.10

ip route 1.1.1.0 255.255.255.255 2.2.2.10

access-list 101 permit ip host 2.2.2.1 host 10.1.1.1

Note : Even through traffic is sent from R1 it will be encrypted only from ASA to R2. Till then it will be only sent in clear text

ASA 8.4 IPSEC VPN CONFIG

Crypto ikev2 policy 10

Authentication pre-share

Encryption 3des

Hash md5

Group 2

Lifetime 84600

Tunnel-group 2.2.2.1 type ipsec-l2l

Tunnel-group 2.2.2.1 ipsec-attributes

Ikev1 pre-shared-key cisco

Lifetime 84600

Crypto ipsec ikev1 transform-set TSET esp-3des esp-sha-hmac

Crypto map CMAP 10 match address 101

Crypto map CMAP 10 set peer 2.2.2.1

Crypto map CMAP 10 set ikev1 transform-set TSET

Crypto isakmp identity address

Crypto ikev1 enable OUTSIDE

Crypto map CMAP interface OUTSIDE