IPSEC SITE TO SITE VPN HAIRPINNING PASSING THROUGH ASA2 & TERMINATING ON ASA1

ASSIGNMENT - 5

TASK:

- Configure IPSEC L2L VPN between R2 and ASA1 such that lo0 of R2 and lo0 of R1 are protected

- You are allowed to open icmp traffic between any zone to any zone only in ASA2 & ASA1 before the VPN is formed to just test the connectivity ( after the tunnel formation the ACL’s should be removed from the ASA)

- Configure IPSEC L2L VPN between R3 and ASA1 such that lo0 of R3 and lo0 of R1 are protected

- In the already established VPN tunnel lo0 of R2 & lo0 of R3 should be protected.

SOLUTION:

R1

int lo0

ip add 1.1.1.1 255.255.255.255

int fa0/0

ip add 10.1.1.1 255.255.255.0

no shut

ip route 2.2.2.2 255.255.255.255 10.1.1.10

ip route 3.3.3.3 255.255.255.255 10.1.1.10

R2

int lo0

ip add 2.2.2.2 255.255.255.255

int fa0/0

ip add 40.1.1.1 255.255.255.0

no shut

ip route 20.1.1.0 255.255.255.0 40.1.1.10

ip route 1.1.1.1 255.255.255.255 40.1.1.10

ip route 3.3.3.3 255.255.255.255 40.1.1.10 - Hairpinning

access-list 101 permit ip host 2.2.2.2 host 1.1.1.1

access-list 101 permit ip host 2.2.2.2 host 3.3.3.3 - Hairpinning

crypto isakmp policy 10

encr aes 256

hash sha

authentication pre-share

group 2

lifetime 86400

crypto isakmp key cisco address 20.1.1.10

crypto ipsec transform-set TSET esp-3des esp-md5-hmac

mode tunnel

crypto map CMAP 10 ipsec-isakmp

set peer 20.1.1.10

set transform-set TSET

match address 101

int fa0/0

crypto map CMAP

R3

int lo0

ip add 3.3.3.3 255.255.255.255

int fa0/0

ip add 50.1.1.1 255.255.255.0

no shut

ip route 20.1.1.0 255.255.255.0 50.1.1.10

ip route 1.1.1.1 255.255.255.255 50.1.1.10

ip route 2.2.2.2 255.255.255.255 50.1.1.10 - Hairpinning

access-list 102 permit ip host 3.3.3.3 host 1.1.1.1

access-list 102 permit ip host 3.3.3.3 host 2.2.2.2 - Hairpinning

crypto isakmp policy 10

encr aes 256

hash sha

authentication pre-share

group 2

lifetime 86400

crypto isakmp key ccie address 20.1.1.10

crypto ipsec transform-set TSET esp-3des esp-md5-hmac

mode tunnel

crypto map CMAP 10 ipsec-isakmp

set peer 20.1.1.10

set transform-set TSET

match address 102

int fa0/0

crypto map CMAP

R4

int fa0/0

ip add 30.1.1.1 255.255.255.0

no shut

int fa0/1

ip add 20.1.1.1 255.255.255.0

no shut

ip route 40.1.1.0 255.255.255.0 30.1.1.10

ip route 50.1.1.0 255.255.255.0 30.1.1.10

ASA1

interface e0/0

nameif OUTSIDE

security-level 0

ip address 20.1.1.10 255.255.255.0

no shut

interface e0/1

nameif INSIDE

security-level 100

ip address 10.1.1.10 255.255.255.0

no shut

route INSIDE 1.1.1.1 255.255.255.255 10.1.1.1

route OUTSIDE 40.1.1.0 255.255.255.0 20.1.1.1

route OUTSIDE 2.2.2.2 255.255.255.255 20.1.1.1

route OUTSIDE 50.1.1.0 255.255.255.0 20.1.1.1

route OUTSIDE 3.3.3.3 255.255.255.255 20.1.1.1

TUNNEL R2-ASA1

access-list 101 permit ip host 1.1.1.1 host 2.2.2.2

crypto isakmp policy 10

authentication pre-share

encryption aes-256

hash sha

group 2

lifetime 86400

exit

tunnel-group 40.1.1.1 type ipsec-l2l

tunnel-group 40.1.1.1 ipsec-attributes

pre-shared-key cisco

exit

crypto ipsec transform-set TSET esp-3des esp-md5-hmac

crypto map CMAP 10 ipsec-isakmp

crypto map CMAP 10 set peer 40.1.1.1

crypto map CMAP 10 match address 101

crypto map CMAP 10 set transform-set TSET

crypto isakmp enable OUTSIDE

crypto map CMAP interface OUTSIDE

TUNNEL R3-ASA1

access-list 102 permit ip host 1.1.1.1 host 3.3.3.3

tunnel-group 50.1.1.1 type ipsec-l2l

tunnel-group 50.1.1.1 ipsec-attributes

pre-shared-key ccie

exit

crypto map CMAP 20 ipsec-isakmp

crypto map CMAP 20 set peer 50.1.1.1

crypto map CMAP 20 match address 102

crypto map CMAP 20 set transform-set TSET

TUNNEL HAIRPINNING

access-list 101 permit ip host 3.3.3.3 host 2.2.2.2

access-list 102 permit ip host 2.2.2.2 host 3.3.3.3

same-security-traffic permit intra-interface

ASA2

interface e0/0

nameif OUTSIDE1

security-level 0

ip address 40.1.1.10 255.255.255.0

no shut

interface e0/1

nameif OUTSIDE2

security-level 0

ip address 50.1.1.10 255.255.255.0

no shut

interface e0/2

nameif INSIDE

security-level 100

ip address 30.1.1.10 255.255.255.0

no shut

same-security-traffic permit inter-interface

route INSIDE 20.1.1.0 255.255.255.0 30.1.1.1

access-list OUT1-IN extended permit esp host 40.1.1.1 host 20.1.1.10

access-list OUT1-IN extended permit udp host 40.1.1.1 host 20.1.1.10 eq 500

access-group OUT1-IN in in OUTSIDE1

access-list OUT2-IN extended permit esp host 50.1.1.1 host 20.1.1.10

access-list OUT2-IN extended permit udp host 50.1.1.1 host 20.1.1.10 eq 500

access-group OUT2-IN in in OUTSIDE2