IPSEC SITE TO SITE VPN HAIRPINNING PASSING THROUGH ASA2 MULTI MODE WITH SUB_INTERFACES & ASA1 TRANSPARENT MODE TERMINATING ON R1

ASSIGNMENT - 7

R1

crypto isakmp policy 10

encr aes

authentication pre-share

group 2

crypto isakmp key cisco address 40.1.1.1

crypto isakmp key ccie address 50.1.1.1

crypto ipsec transform-set TSET esp-3des esp-md5-hmac

crypto map CMAP 10 ipsec-isakmp

set peer 40.1.1.1

set transform-set TSET

match address 101

crypto map CMAP 20 ipsec-isakmp

set peer 50.1.1.1

set transform-set TSET

match address 102

interface Loopback0

ip address 1.1.1.1 255.255.255.255

interface FastEthernet0/0

ip address 10.1.1.1 255.255.255.0

crypto map CMAP

ip route 2.2.2.2 255.255.255.255 10.1.1.2

ip route 3.3.3.3 255.255.255.255 10.1.1.2

ip route 40.1.1.0 255.255.255.0 10.1.1.2

ip route 50.1.1.0 255.255.255.0 10.1.1.2

access-list 101 permit ip host 1.1.1.1 host 2.2.2.2

access-list 101 permit ip host 3.3.3.3 host 2.2.2.2

access-list 102 permit ip host 1.1.1.1 host 3.3.3.3

access-list 102 permit ip host 2.2.2.2 host 3.3.3.3

R2

crypto isakmp policy 10

encr aes

authentication pre-share

group 2

crypto isakmp key cisco address 10.1.1.1

crypto ipsec transform-set TSET esp-3des esp-md5-hmac

crypto map CMAP 10 ipsec-isakmp

set peer 10.1.1.1

set transform-set TSET

match address 101

interface Loopback0

ip address 2.2.2.2 255.255.255.255

interface FastEthernet0/0

ip address 40.1.1.1 255.255.255.0

crypto map CMAP

ip route 1.1.1.1 255.255.255.255 40.1.1.10

ip route 3.3.3.3 255.255.255.255 40.1.1.10

ip route 10.1.1.0 255.255.255.0 40.1.1.10

ip route 20.1.1.0 255.255.255.0 40.1.1.10

access-list 101 permit ip host 2.2.2.2 host 1.1.1.1

access-list 101 permit ip host 2.2.2.2 host 3.3.3.3

R3

crypto isakmp policy 10

encr aes

authentication pre-share

group 2

crypto isakmp key ccie address 10.1.1.1

crypto ipsec transform-set TSET esp-3des esp-md5-hmac

crypto map CMAP 10 ipsec-isakmp

set peer 10.1.1.1

set transform-set TSET

match address 102

interface Loopback0

ip address 3.3.3.3 255.255.255.255

interface FastEthernet0/0

ip address 50.1.1.1 255.255.255.0

crypto map CMAP

ip route 1.1.1.1 255.255.255.255 50.1.1.10

ip route 2.2.2.2 255.255.255.255 50.1.1.10

ip route 10.1.1.0 255.255.255.0 50.1.1.10

ip route 30.1.1.0 255.255.255.0 50.1.1.10

access-list 102 permit ip host 3.3.3.3 host 1.1.1.1

access-list 102 permit ip host 3.3.3.3 host 2.2.2.2

R4

interface FastEthernet0/0

no shutdown

interface FastEthernet0/0.10

encapsulation dot1Q 10

ip address 20.1.1.1 255.255.255.0

interface FastEthernet0/0.20

encapsulation dot1Q 20

ip address 30.1.1.1 255.255.255.0

ip route 40.1.1.0 255.255.255.0 20.1.1.10

ip route 50.1.1.0 255.255.255.0 30.1.1.10

ASA1

interface BVI1

ip address 10.1.1.3 255.255.255.0

interface GigabitEthernet0

nameif OUTSIDE

bridge-group 1

security-level 0

interface GigabitEthernet1

nameif INSIDE

bridge-group 1

security-level 100

access-list OUT-IN extended permit ip any any

access-list OUT-IN extended permit esp any any

access-list OUT-IN extended permit udp any any

access-group OUT-IN in interface OUTSIDE

ASA2

interface GigabitEthernet1

no shutdown

interface GigabitEthernet1.10

vlan 10

interface GigabitEthernet1.20

vlan 20

admin-context admin

context c1

allocate-interface GigabitEthernet0

allocate-interface GigabitEthernet1.10

config-url disk0:/c1.cfg

context c2

allocate-interface GigabitEthernet1.20

allocate-interface GigabitEthernet2

config-url disk0:/c2.cfg

ASA/C1

interface GigabitEthernet0

nameif OUTSIDE1

security-level 0

ip address 40.1.1.10 255.255.255.0

interface GigabitEthernet1.10

nameif INSIDE

security-level 100

ip address 20.1.1.10 255.255.255.0

access-list OUT1-IN extended permit ip any any

access-list OUT1-IN extended permit esp any any

access-list OUT1-IN extended permit udp any any

access-group OUT1-IN in interface OUTSIDE1

route INSIDE 10.1.1.0 255.255.255.0 20.1.1.1 1

ASA/C2

interface GigabitEthernet2

nameif OUTSIDE2

security-level 0

ip address 50.1.1.10 255.255.255.0

interface GigabitEthernet1.20

nameif INSIDE

security-level 100

ip address 30.1.1.10 255.255.255.0

access-list OUT2-IN extended permit ip any any

access-list OUT2-IN extended permit esp any any

access-list OUT2-IN extended permit udp any any

access-group OUT2-IN in interface OUTSIDE2

route INSIDE 10.1.1.0 255.255.255.0 30.1.1.1 1