IPSEC SITE TO SITE VPN USING CERTIFICATE AUTHORITY & RSA SIGNATURES

ASSIGNMENT - 8

CA

crypto pki server IOSCA

issuer-name CN=CCIE OU=IPSECVPN S=BANGALORE

grant auto

lifetime certificate 4

lifetime ca-certificate 5

crypto pki trustpoint IOSCA

revocation-check crl

rsakeypair IOSCA

crypto pki certificate chain IOSCA

certificate ca 01 nvram:CCIEOUIPSECV#1CA.cer

interface FastEthernet0/0

ip address 10.1.1.1 255.255.255.0

ip route 20.1.1.0 255.255.255.0 10.1.1.5

ip route 30.1.1.0 255.255.255.0 10.1.1.5

ip route 40.1.1.0 255.255.255.0 10.1.1.5

ntp authentication-key 1 md5 030752180500 7

ntp authenticate

ntp trusted-key 1

ntp clock-period 17179740

ntp server 10.1.1.5

NTP

interface FastEthernet0/0

ip address 20.1.1.5 255.255.255.0

interface FastEthernet0/1

ip address 10.1.1.5 255.255.255.0

ip route 30.1.1.0 255.255.255.0 20.1.1.10

ip route 40.1.1.0 255.255.255.0 20.1.1.10

ntp authentication-key 1 md5 1511021F0725 7

ntp authenticate

ntp trusted-key 1

ntp master 1

R1

crypto pki trustpoint IOSCA

enrollment url http://10.1.1.1:80

ip-address 30.1.1.1

revocation-check none

crypto pki authenticate IOSCA – cli command only

crypto pki enroll IOSCA – cli command only

crypto pki certificate chain IOSCA

certificate 02 nvram:CCIEOUIPSECV#2.cer

certificate ca 01 nvram:CCIEOUIPSECV#1CA.cer

crypto isakmp policy 10

auth rsa-sig

crypto ipsec transform-set TSET esp-3des esp-sha-hmac

crypto map CMAP 10 ipsec-isakmp

set peer 40.1.1.1

set transform-set TSET

match address 101

interface Loopback0

ip address 1.1.1.1 255.255.255.255

interface FastEthernet0/0

ip address 30.1.1.1 255.255.255.0

crypto map CMAP

ip route 2.2.2.2 255.255.255.255 30.1.1.10

ip route 10.1.1.0 255.255.255.0 30.1.1.10

ip route 20.1.1.0 255.255.255.0 30.1.1.10

ip route 40.1.1.0 255.255.255.0 30.1.1.10

access-list 101 permit ip host 1.1.1.1 host 2.2.2.2

ntp authentication-key 1 md5 05080F1C2243 7

ntp authenticate

ntp trusted-key 1

ntp clock-period 17179750

ntp server 20.1.1.5

R2

crypto pki trustpoint IOSCA

enrollment url http://10.1.1.1:80

ip-address 30.1.1.1

revocation-check none

crypto pki authenticate IOSCA – cli command only

crypto pki enroll IOSCA – cli command only

crypto pki certificate chain IOSCA

certificate 08 nvram:CCIEOUIPSECV#8.cer

certificate ca 01 nvram:CCIEOUIPSECV#1CA.cer

crypto isakmp policy 10

auth rsa-sig

crypto ipsec transform-set TSET esp-3des esp-sha-hmac

crypto map CMAP 10 ipsec-isakmp

set peer 30.1.1.1

set transform-set TSET

match address 101

interface Loopback0

ip address 2.2.2.2 255.255.255.255

interface FastEthernet0/0

ip address 40.1.1.1 255.255.255.0

crypto map CMAP

ip route 1.1.1.1 255.255.255.255 40.1.1.10

ip route 10.1.1.0 255.255.255.0 40.1.1.10

ip route 20.1.1.0 255.255.255.0 40.1.1.10

ip route 30.1.1.0 255.255.255.0 40.1.1.10

access-list 101 permit ip host 2.2.2.2 host 1.1.1.1

ntp authentication-key 1 md5 13061E010803 7

ntp authenticate

ntp trusted-key 1

ntp clock-period 17179849

ntp server 20.1.1.5