GRE OVER IPSEC USING ESP IN TRANSPORT MODE PASSING THROUGH ASA RUNNING EIGRP

ASSIGNMENT - 9

R1

crypto isakmp policy 10

authentication pre-share

crypto isakmp key cisco123 address 20.1.1.1

crypto ipsec transform-set TSET esp-3des esp-sha-hmac

mode transport

crypto ipsec profile GRE-IPSEC

set transform-set TSET

interface Loopback0

ip address 1.1.1.1 255.255.255.255

interface Tunnel0

ip address 172.16.1.1 255.255.255.0

tunnel source FastEthernet0/0

tunnel destination 20.1.1.1

tunnel key 123

tunnel protection ipsec profile GRE-IPSEC

interface FastEthernet0/0

ip address 10.1.1.1 255.255.255.0

router eigrp 100

network 1.1.1.1 0.0.0.0

network 10.1.1.0 0.0.0.255

network 172.16.1.0 0.0.0.255

no auto-summary

ip route 20.1.1.0 255.255.255.0 10.1.1.10

R2

crypto isakmp policy 10

authentication pre-share

crypto isakmp key cisco123 address 10.1.1.1

crypto ipsec transform-set TSET esp-3des esp-sha-hmac

mode transport

crypto ipsec profile GRE-IPSEC

set transform-set TSET

interface Loopback0

ip address 2.2.2.2 255.255.255.255

interface Tunnel0

ip address 172.16.1.2 255.255.255.0

tunnel source FastEthernet0/0

tunnel destination 10.1.1.1

tunnel key 123

tunnel protection ipsec profile GRE-IPSEC

interface FastEthernet0/0

ip address 20.1.1.1 255.255.255.0

router eigrp 100

network 2.2.2.2 0.0.0.0

network 20.1.1.0 0.0.0.255

network 172.16.1.0 0.0.0.255

no auto-summary

SW1

interface range fa1/0 – 1

switchport mode access

switchport access vlan 10

SW2

interface range fa1/0 – 1

switchport mode access

switchport access vlan 20

ASA1

interface Ethernet0/0

nameif INSIDE

security-level 100

ip address 10.1.1.10 255.255.255.0

interface Ethernet0/1

nameif OUTSIDE

security-level 0

ip address 20.1.1.10 255.255.255.0

access-list OUT-IN extended permit gre host 20.1.1.1 host 10.1.1.1

access-list OUT-IN extended permit icmp any any

access-list OUT-IN extended permit esp host 20.1.1.1 host 10.1.1.1

Note: We cannot advertise the tunnel network and the physical network using the same protocol. So we advertise the physical network using static routes and the tunnel networks using EIGRP

GRE over IPSEC can operate in both tunnel mode and transport mode but transport mode is most preferred solution as it eliminates the 20 bytes additional header produced in tunnel mode.